Cleish Village Hall

Fire Safety Risk Assessment

A Safety Guide for Users of the Hall

Review Date: 23rd December 2023

Introduction

This fire risk assessment is holistic and dynamic, recognising that the use of the building and its occupancy is rarely static. The emphasis has been placed upon simply raising a number of relevant issues and asking whether or not they present any significant risks.

It is recognised that in common with many other evaluation schemes, risk assessments will be affected by natural variations in perceptions and judgements. In cases of doubt it may be necessary to seek further advice or a second opinion.

The five steps undertaken to complete this fire risk assessment include:-

- Look for fire hazards

- Decide who might be harmed and how

- Evaluate the risks and decide whether the existing precautions are adequate or whether more should be done.

- Record the findings

- Review the assessment and revise it as necessary.

Description of Premises

Cleish Village Hall was built in 1928 and extended and renovated in 1996.The Hall is a registered charity and is run by a Management Committee of local volunteers. The committee consists of elected members of the public.

The facilities comprise the Main Hall, the Small Meeting Room, Kitchen, an entrance lobby, equipment store room and separate men and women’s disabled toilets. The Hall is available to hire for both private and public events. The Main Hall can hold up to 100 people and the Small Meeting Room 20. There are emergency exits leading directly out of the building from the Main Hall and the Small Meeting Room.

The Hall is hired by a range of voluntary organisations, private individuals and statutory organisations for a variety of activities, some of which are licensable. A written agreement between the Village Hall and the Hirer forms the basis of the Hiring arrangement. The Hire Agreement and Standard Conditions of Hire draw the hirers’ attention to the health and safety and licensing obligations that the booking places upon them. Hirers are expected to provide attendants to meet the requirements of licensing conditions. Hirers are responsible for meeting the obligations of the licence.

Principal Uses of the Village Hall

The Village Hall is used for a wide variety of functions. These cover, but are not limited to, in-door sports events, dog lessons, cabarets, plays, film shows, private parties and charity fund raisers.

The Village Hall Committee encourages all regular users of the Hall to carry out their own Fire Risk Assessment, highlighting the areas that pertain to their own group of users and their activities. The Village Hall Committee encourages hirers to make themselves aware of the exit routes, the fire fighting equipment and the plan of the Hall detailing the location of the equipment and the escape routes. The plan is available to all Hall users and is also on the notice boards of the Main Hall.

People at Risk of Fire

Staff: The Village Hall Committee does not employ any staff

Trades People: The committee employs local trades people on both an ad-hoc and scheduled basis for the purposes of cleaning, safety checks and building maintenance. The cleaner has a key to the building and is generally on their own when working. Local trades people who are familiar with the building may gain access to the Hall by borrowing a key from the bookings secretary or other committee member, while others are always escorted by one or more committee members.

Hall Users (up to 100 maximum inside at any time): These will generally be familiar with the Hall. Those that are not will find the simple layout easy to navigate and assimilate. The Main Hall area is open plan and has 2 easily identifiable fire exits and the Small Meeting Room has 2 easily identifiable fire exits. Emergency lighting is installed over fire doors and around the Hall, and this is tested regularly. A fire alert and detection system with audible sounders is installed, and this is tested regularly. There are fire doors exit directly outside at the rear of the Main Hall. The main double entrance doors to the Hall must be kept unlocked whenever the Hall is in use for a function.

Disabled Persons: At a typical function there may possibly be a limited number of disabled persons. It is the responsibility of Hall hirers to ensure disabled persons are given adequate assistance in the event of an evacuation. Emergency exit doors in Main Hall are on a level with the paths outside, which are paving slabs, so there are no barriers to wheel chairs.

Children: It is the responsibility of hirers to ensure all children within the Hall are supervised and that they are given adequate assistance in the event of an evacuation. Primary Hall user groups, like the School, are expected to have carried out their own Fire Risk Assessment.

Other Members of the Public (who are not using the Hall): Members of the public may walk past the front of the Hall.

Possible Causes of Fire

| Source of Ignition | Source of Fuel | Source of Oxygen |
| --- | --- | --- |
| Faulty electrical system in mains power supply, ring mains or lighting circuits. | Any material close to the source of the fault (e.g. the cupboard in which the consumer unit is located, skirting boards, plaster board, wood panelling). | Open to the air |
| Oil leak in supply to boilers. | Any combustible materials close to the boilers | Open to the air |
| Faulty portable electrical equipment (e.g. kettles, toasters, radios, etc) | The portable device itself and then any combustible materials close to it | Open to the air |
| Kitchen appliances (i.e. hob, oven, fridge, kettle, microwave) | The appliance itself and then any combustible materials close to it | Open to the air |
| Smoking | Material in waste bins or any other areas where cigarettes may be left or disposed of. | Open to the air |
| Cooking accidents | Hot oil igniting on the hob or under a grill. Food burning in the oven or on the hob. | Open to the air |
| Arson | Could be any source | Open to the air |
| Candles and other naked flames | The candles and combustible materials close to them. | Open to the air |

Control Measures

| Fire hazard | Likelihood 1=low 5=high | Risk to people from fire hazard | Measures to avoid or reduce |  |
| --- | --- | --- | --- | --- |
|  |  |  | The hazard | Risk to people |
| Fire inside the Hall | 2 | Smoke inhalation. Burns. Death. Particular attention to be paid to people who are sleeping overnight or attending large events. | Ensure sufficient and correct fire fighting equipment is available and that a qualified practitioner inspects it at least once per year and provides advice on changing or moving locations as appropriate. The location of fire-fighting equipment and exits are clearly marked on a plan of the village Hall, which is published on the notice boards. A test of continued illumination in event of a power failure will be done quarterly. Ensure that village hall furnishings are made of a non-combustible material or a material that has been Fire Proofed to the standard applicable at the time of purchase. This includes window curtains, chairs, etc. Hall users will be asked to remove all their rubbish when they leave the building (i.e. emptying any bins) | Ensure Hall users/hirers know how to evacuate the building, where the fire-fighting equipment is located and where the assembly points are. A fire alert system has been installed so that an alarm can be raised if a fire is discovered. Ensure the hirers know that they must organise a fire martial to check the building has been vacated after alarm and that the fire brigade has been called out (as appropriate). Ensure all Fire Exit doors are checked for illumination, ease of opening and that the Emergency Exit Routes are clear on a weekly basis. The Management Committee tests the ease of access to and through the escape routes and to the assembly points at least once per year and logs the results. Users will be made aware of their responsibilities under the Premises License (via the Booking process). Regular User Groups will be advised that they should carry out their own Risk Assessment and that a copy should be lodged with the Management Committee. |
| Fire on the premises outside the Hall. | 1 | Evacuating people towards the source of the fire |  | Ensure hirers understand and accept their responsibility to evacuate the building in the event of a fire and to allocate a responsible person to ensure the evacuation is coordinated properly and safely. |
| Faulty electrics in mains power supply, ring mains, lighting circuits, kitchen appliances or portable devices. | 1 | Electrocution while attempting to put out the fire with water. | A qualified practitioner checks the fixed wiring once every 3 years and portable appliances are checked every year. A Certificate of Inspection will be obtained for each inspection. | Ensure suitable (not water) fire fighting equipment is available close to the likely sources of electrical fires. |
| Leak in supply to boilers | 1 | Death from explosive combustion and associated damage. | Ensure that the appliances are maintained and tested at least once per year. A safety inspection certificate to be obtained after each service. |  |
| Smoking | 3 | Smoke inhalation. Burns. | A No Smoking policy enforced throughout the premises. No Smoking signs strategically placed throughout the building. |  |
| Cooking accidents | 3 | Smoke inhalation. Burns. | Ensure users are aware of appropriate safety precautions concerning the use of the cooker. No deep fat fryers allowed. Ensure there is a fire blanket available close to the hob, for putting out oil fires. |  |
| Children playing with matches or other combustibles | 2 | Smoke inhalation. Burns. |  | Ensure hirers are aware of their responsibility to supervise children while inside the Hall. |
| Arson | 1 | Smoke inhalation. Burns. | Ensure Hall hirers are aware of their responsibility for the Hall during their occupation and take reasonable steps to prevent wilful damage and lock up on departure. | Wherever possible, locking mechanisms for ensuring internal safety without comprising emergency routes are fitted. |

Plan of the Hall

USER FIRE ACTION

In these premises the fire warning is given by the continuous sounding of a bell

Use the nearest break glass point to alert users of a fire.

ON discovering a fire:-

- Raise the alarm by operating the nearest fire alarm point.

- Tackle the out break with a fire extinguisher but only if it is safe to do so, other wise leave the building and proceed to the assemble point at:-

Across the road, opposite the front of the building

ON hearing a Warning of fire:-

- Alert all persons under your charge – or make sure they have been alerted.

- All persons should evacuate the premises quickly but  calmly by the nearest exit and proceed to the assemble point at:-

Across the road, opposite the front of the building

- Do not delay the departure by collecting coats or other personal belongings

- Before leaving, check cloakrooms and toilets to ensure that all persons have left the premises.

- Close all doors of rooms and those through which you pass on leaving the building.

- Ensure that the fire brigade is called immediately you hear the fire warning – Dial 999 and give the full postal address of the affected premises when connected with the fire brigade operator.

Cleish Village Hall,

Cleish, KY13 0LR

- Do not re-enter the building until a fire brigade officer has stated that it is safe to do so.